Q. Demonstrate DHCP server in Packet Tracer.

Aim: To write code to demonstrate DHCP in packet tracer.

Steps:

- One  router , switch and two PCs

- Connect both the 2 PCs to switch and connect switch to the router.

- Give router a IP address then PC0 and PC1 click the radio button of DHCP in the IP configuration panel,the values will be assigned automatically.

Topology:

Code:

Router>enable

Router#configure terminal

Router(config)#interface FastEthernet0/0

Router(config-if)#ip address  192.168.1.1 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#ip dhcp pool mypool

Router(dhcp-config)#network 192.163.1.0 255.255.255.0

Router(dhcp-config)#default-router 192.168.1.1

Router(dhcp-config)#exit

Output:

Conclusion: The above network topology has been executed successfully.