# Q. Demonstrate the RIP Routing in Packet tracer.

Aim: To write code to demonstrate rip routing in packet tracer.

Steps:

- In packet tracer place 2 PCs and 2 Routers.

- Connect Router0 with PC0 via fastethernet0/0 using cross-copper cord and Router1 with PC1 via fastethernet0/0 using cross-copper cord.

- Connect both router via fastethernet 0/1 using cross-copper cord.

- Set the IP address and default gatway of both the pcs in ip configuration of both the pcs respectively.

Topology:

Code:

Router0:

Router>enable

Router#configure terminal

Router(config)#interface FastEthernet0/0

Router(config-if)#ip address  192.168.1.3 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface FastEthernet0/1

Router(config-if)#ip address 192.168.3.2 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config-router)#router rip

Router(config-router)#network 192.168.1.0

Router(config-router)#network 192.168.3.0

Router(config-router)#exit

Router 1:

Router>enable

Router#configure terminal

Router(config)#interface FastEthernet0/0

Router(config-if)#ip address 192.168.2.3 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface FastEthernet0/1

Router(config-if)#ip address 192.168.3.3 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config-router)#router rip

Router(config-router)#network 192.168.2.0

Router(config-router)#network 192.168.3.0

Router(config-router)#exit.

Verification: On Any One PC.

Router# show ip protocol

Router# show ip route

Router# show ip route rip

Output:

Conclusion: The above network topology has been executed successfully.