# Q. Demonstrate the Static Routing in Packet tracer.

Aim: To write code to demonstrate static routing in packet tracer.

Steps:

- In packet tracer place 2 PCs and 2 Routers.

- Connect Router0 with PC0 via fastethernet0/0 using cross-copper cord and Router1 with PC1 via fastethernet0/0 using cross-copper cord.

- Connect both router via fastethernet 0/1 using cross-copper cord.

- Set the IP address and default gatway of both the pcs in ip configuration of both the pcs respectively.

Topology:

Code:

Router0:

Router>enable

Router#configure terminal

Router(config)#interface FastEthernet0/0

Router(config-if)#ip address 10.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface FastEthernet0/1

Router(config-if)#ip address 20.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#ip route 40.0.0.0 255.0.0.0 20.0.0.2

Router(config)#exit

Router 1:

Router>enable

Router#configure terminal

Router#configure terminal

Router(config)#interface FastEthernet0/0

Router(config-if)#ip address 20.0.0.2 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface FastEthernet0/1

Router(config-if)#ip address 40.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#ip route 10.0.0.0 255.0.0.0 20.0.0.1

Router(config)#exit.

Verification: In Both PCs Command Prompt

PC1:

C:\ipconfig

C:\ping 10.0.0.1

C:\ping 20.0.0.1

C:\ping 20.0.0.2

C:\ping 40.0.0.1

C:\ping 40.0.0.2

PC2:

C:\ipconfig

C:\ping 40.0.0.1

C:\ping 20.0.0.2

C:\ping 20.0.0.1

C:\ping 10.0.0.1

C:\ping 10.0.0.2

Output:

Pc0:

Pc1:

Conclusion: The above network topology has been executed successfully.