Q. Demonstrate Telnet in Packet Tracer.

Aim: To write code to demonstrate Telnet in packet tracer.

Steps:

- One router , switch and  PC

- Connect the PC to switch and connect switch to the router using copper straight-through cable.

- Do router,switch interface up.

Topology:

Code:

Switch:

Switch>enable

Switch# configure terminal

Switch(config)#interface fastethernet0/0

Switch(config-if)# ip address  192.168.1.2 255.255.255.0

Switch(config-if)#no shutdown

Switch(config-if)#exit

Switch(config)#line vty04

Switch(config-line)#password cisco

Switch(config-line)#login

Switch(config-line)#end

Switch#write memory

Router:

Router>enable

Router# configure terminal

Router(config)#interface fastethernet0/0

Router(config-if)# ip address  192.168.1.1 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#line vty04

Router(config-line)#password cisco

Router(config-line)#login

Router(config-line)#end

Verification:

In cmd of pc:

>telnet 192.168.1.1

>telnet 192.168.1.2

Output:

Conclusion: The above network topology has been executed successfully.