Study of Networking Device

Q1.  Identify the  network address & subnet mask for the ip address 192.168.1.0

Given ip address is 192.168.1.0

The subnet mask is 255.255.255.0  as it is belong to class C

In Binary Form:-

11111111 .11111111 .11111111.00000000 In this the 1st 24 bits represent host part.

Q2. Create 4 Subnets:-

To create 4 subnets we need to binary two bits from the host part.

2^2=4

So the new subnet mask is  255.255.255.192

Binary Form:-

11111111.11111111.11111111.11000000

so  26 bits for the network part 6 bits for host part. Total no. of subnet 4.

[ combining 2 or more subnet will give us the Supernet.]

Total no. Of useable ip address in each subnet is 64- -2 because  1 address is for network and 1 for broadcast, so subtracting them.  62 IP address are useable.

CIDR format:-classless InterDomain Representaion.

New Subnet Mask:- 255.255.255.192/126

The 1st subnet mask is 192.168.1.0/126

The  2nd subnet mask is 192.168.1.64/126

Subnet 1=192.168.1.0 ---> Network address

Useable host address 192.168.1.62/126

Broadcast address 192.168.1.63/26

Subnet 2 192.168.1.64

1st  host address 192.168.1.65

Last host address                     192.168.1.126/26

Broadcast address                  192.168.1.127

Subnet 3                                        192.168.1.128

Last address is                           192.168.1.190

15th address is                           192.168.1.129

Subnet 4                                        192.168.1.192

Last address is                            192.168.1.254

Broadcast address                     192.168.1.255

Q3. A network address     192.168.1.0/24  create 3 subnet with subnet 1 corristing if so subnet 2 with 20, and subnet 3 with 10.

(Since the no. Of host bit is determine of the ip address. So ip address formula to calculate no. Of bits required to satisfy  each subnet ip address is 2^n/   no.  of ip address required.

Where n is the no. of bits and  +2 represent n|6)

Subnet 1 of  50 ip address

Ex:- 2^6 = 64    with 62 useable ip address we need 6 host bits.

Subnet mask is 255.255.255.192/26

2^5  = 32 ,30 useable

Subnet mask is 255.255.255.224|27

2^4=16

Subnet   255.255.255.140|28

Subnet 1

Subnet  mask:                                                     255.255.255.192|26

Network address                                                192.168.1.0|26

Host address                                            192.168.1.1|26

Last host address                                    192.168.1.50|26

Useable ip address                           62//

Broadcast                                              192.168.1.63|26

# Q. Demonstrate the Static Routing in Packet tracer.

Aim: To write code to demonstrate static routing in packet tracer.

Steps:

- In packet tracer place 2 PCs and 2 Routers.

- Connect Router0 with PC0 via fastethernet0/0 using cross-copper cord and Router1 with PC1 via fastethernet0/0 using cross-copper cord.

- Connect both router via fastethernet 0/1 using cross-copper cord.

- Set the IP address and default gatway of both the pcs in ip configuration of both the pcs respectively.

Topology:

Code:

Router0:

Router>enable

Router#configure terminal

Router(config)#interface FastEthernet0/0

Router(config-if)#ip address 10.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface FastEthernet0/1

Router(config-if)#ip address 20.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#ip route 40.0.0.0 255.0.0.0 20.0.0.2

Router(config)#exit

Router 1:

Router>enable

Router#configure terminal

Router#configure terminal

Router(config)#interface FastEthernet0/0

Router(config-if)#ip address 20.0.0.2 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface FastEthernet0/1

Router(config-if)#ip address 40.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#ip route 10.0.0.0 255.0.0.0 20.0.0.1

Router(config)#exit.

Verification: In Both PCs Command Prompt

PC1:

C:\ipconfig

C:\ping 10.0.0.1

C:\ping 20.0.0.1

C:\ping 20.0.0.2

C:\ping 40.0.0.1

C:\ping 40.0.0.2

PC2:

C:\ipconfig

C:\ping 40.0.0.1

C:\ping 20.0.0.2

C:\ping 20.0.0.1

C:\ping 10.0.0.1

C:\ping 10.0.0.2

Output:

Pc0:

Pc1:

Conclusion: The above network topology has been executed successfully.

# Q. Demonstrate the OSPF Routing in Packet tracer.

Aim: To write code to demonstrate ospf routing in packet tracer.

Steps:

- In packet tracer place 2 PCs and 2 Routers.

- Connect Router0 with PC0 via fastethernet0/0 using cross-copper cord and Router1 with PC1 via fastethernet0/0 using cross-copper cord.

- Connect both router via fastethernet 0/1 using cross-copper cord.

- Set the IP address and default gatway of both the pcs in ip configuration of both the pcs respectively.

Topology:

Code:

Router0:

Router>enable

Router#configure terminal

Router(config)#interface FastEthernet0/0

Router(config-if)#ip address  192.168.1.3 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface FastEthernet0/1

Router(config-if)#ip address 192.168.3.2 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config-router)#router ospf1

Router(config-router)#network 192.168.1.0 0.0.0.255 area 0

Router(config-router)#network 192.168.3.0 0.0.0.255 area 0

Router(config-router)#exit

Router 1:

Router>enable

Router#configure terminal

Router(config)#interface FastEthernet0/0

Router(config-if)#ip address 192.168.2.3 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface FastEthernet0/1

Router(config-if)#ip address 192.168.3.3 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config-router)#router ospf2

Router(config-router)#network 192.168.2.0 0.0.0.255 area 0

Router(config-router)#network 192.168.3.0 0.0.0.255 area 0

Router(config-router)#exit.

Verification: On Any One PC.

Router# show ip ospf neighbor

Router# show ip route ospf

Router# show ip protocol

Output:

Conclusion: The above network topology has been executed successfully.

# Q. Demonstrate the RIP Routing in Packet tracer.

Aim: To write code to demonstrate rip routing in packet tracer.

Steps:

- In packet tracer place 2 PCs and 2 Routers.

- Connect Router0 with PC0 via fastethernet0/0 using cross-copper cord and Router1 with PC1 via fastethernet0/0 using cross-copper cord.

- Connect both router via fastethernet 0/1 using cross-copper cord.

- Set the IP address and default gatway of both the pcs in ip configuration of both the pcs respectively.

Topology:

Code:

Router0:

Router>enable

Router#configure terminal

Router(config)#interface FastEthernet0/0

Router(config-if)#ip address  192.168.1.3 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface FastEthernet0/1

Router(config-if)#ip address 192.168.3.2 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config-router)#router rip

Router(config-router)#network 192.168.1.0

Router(config-router)#network 192.168.3.0

Router(config-router)#exit

Router 1:

Router>enable

Router#configure terminal

Router(config)#interface FastEthernet0/0

Router(config-if)#ip address 192.168.2.3 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface FastEthernet0/1

Router(config-if)#ip address 192.168.3.3 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config-router)#router rip

Router(config-router)#network 192.168.2.0

Router(config-router)#network 192.168.3.0

Router(config-router)#exit.

Verification: On Any One PC.

Router# show ip protocol

Router# show ip route

Router# show ip route rip

Output:

Conclusion: The above network topology has been executed successfully.

Q. Demonstrate DHCP server in Packet Tracer.

Aim: To write code to demonstrate DHCP in packet tracer.

Steps:

- One  router , switch and two PCs

- Connect both the 2 PCs to switch and connect switch to the router.

- Give router a IP address then PC0 and PC1 click the radio button of DHCP in the IP configuration panel,the values will be assigned automatically.

Topology:

Code:

Router>enable

Router#configure terminal

Router(config)#interface FastEthernet0/0

Router(config-if)#ip address  192.168.1.1 255.255.255.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#ip dhcp pool mypool

Router(dhcp-config)#network 192.163.1.0 255.255.255.0

Router(dhcp-config)#default-router 192.168.1.1

Router(dhcp-config)#exit

Output:

Conclusion: The above network topology has been executed successfully.

Q. Demonstrate DNS server in Packet Tracer.

Aim: To write code to demonstrate DNS in packet tracer.

Steps:

- One  server , switch and PC

- Connect PC to switch and connect switch to the server.

- Then in server on the DNS service

- Then assign IP address and DNS server

- For pc assign the ip address and DNS server IP address of Server.

Topology:

Code:

- Go to http in server

- There in file named index will appear,make some changes in that file and save it.

- In DNS service,add the name for website and the DNS server address and make the entry of this detail.

- Now, go to PC->Desktop->web browser.

- Put the name of the file in URL and click Go.

- If the topology works successfully then the website will be displayed

Output:

Conclusion: The above network topology has been executed successfully.

Q. Demonstrate HTTP server in Packet Tracer.

Aim: To write code to demonstrate HTTP in packet tracer.

Steps:

- Two server , one switch and PC

- Connect PC to switch and connect switch to both the server.

Topology:

Code:

Server0(DNS server)

Assign IP address:192.168.1.1 , DNS server IP: 192.168.1.1

Server1(Web server)

Assign IP address:192.168.1.2 , DNS server IP: 192.168.1.1

DNS SERVER IP will be the same.

PC0:

Assign IP address:192.168.1.3 , DNS server IP: 192.168.1.1

- In DNS server, select and on DNS.

- Go to http in web server

- There in file named index will appear,make some changes in that file and save it.

- In DNS server,add the name for website and the DNS server address and make the entry of this detail in the DNS column.

- Now, go to PC->Desktop->web browser.

- Put the name of the file in URL and click Go.

- If the topology works successfully then the website will be displayed

Output:

Conclusion: The above network topology has been executed successfully.

Q. Demonstrate FTP server in Packet Tracer.

Aim: To write code to demonstrate FTP in packet tracer.

Steps:

- One server , switch and two PCs

- Connect both the PCs to switch and connect switch to the server.

Topology:

Code:

Server:

- Enable FTP option

- Create username and password

- Click all 5 tick boxes i.e write,read,delete,rename,list.

- Add it.

PC0:

Assign IP address:10.10.10.1 , Default gateway:10.10.10.0

PC1:

Assign IP address:10.10.10.2 , Default gateway:10.10.10.0

PC0:in cmd

C:\>ipconfig

C:\>ftp 10.10.10.0

C:\>username

Password

Logged in

In text editor

- Create a file and name it hi.html

- Save it

Again in cmd:

ftp>put hi.html

ftp>dir

If the file is there in the list the topology is successfully done.

Output:

Conclusion: The above network topology has been executed successfully.

Q. Demonstrate substitution technique in C.

Aim: To write code to demonstrate substitution technique in C.

Description: A substitution technique in cryptography is a method where each element of the plain text is replaced with another symbol, letter or number to form the Cipher text, preserving the original message length but obscuring its content.

Program:

#include <stdio.h>

#include <conio.h>

void main()

{

char str[26];

int i;

clrscr();

printf("Enter plain text: ");

scanf("%s",str);

for(i=0, str[i]!='\0'; i++)

{

str[i] = str[i] + 4;

}

printf("Encrypted text : \n");

printf("Encrypted : %s\n",str);

for(i=0, str[i]!='\0'; i++)

{

str[i] = str[i] - 4;

}

printf("Decrypted text : \n");

printf("Decrypted : %s\n",str);

getch();

}

Output:

Enter plain text : shreya

Encrypted text : +4

Encrypted : wlviec

Decrypted text : -4

Decrypted : shreya

Conclusion: The above program has been executed successfully

Q. Demonstrate DES in python

Aim: To write code to demonstrate DES in python.

Program:

!pip install pycryptodome

from Crypto.Cipher import DES

from Crypto.Util.Padding import pad, unpad

text = "Hello from shreya!"

key_input = "mysecret"

key = key_input.encode()

data = text.encode()

cipher = DES.new(key, DES.MODE_ECB)

padded_data = pad(data, DES.block_size)

encrypted_data = cipher.encrypt(padded_data)

print(" Encrypted (DES):", encrypted_data)

decipher = DES.new(key, DES.MODE_ECB)

decrypted_data = unpad(decipher.decrypt(encrypted_data), DES.block_size)

print(" Decrypted (DES):", decrypted_data.decode())

Output:

Conclusion: The above program has been executed successfully

Q. Demonstrate AES in python

Aim: To write code to demonstrate AES in python.

Program:

!pip install pycryptodome

from Crypto.Cipher import AES

from Crypto.Util.Padding import pad

import binascii

sykey=bytes.fromhex('2b7e151628aeb2a6abf7158809cf4f3c')

originaltext=b"0123456789ABCDEF"

ciphertext=AES.new(sykey,AES.MODE_ECB)

cencrypt=ciphertext.encrypt(originaltext)

print(cencrypt.hex())

ciphertext1=cencrypt.hex()

ctext=AES.new(sykey,AES.MODE_ECB)

dtext=ctext.decrypt(cencrypt)

print(dtext)

Output:

Conclusion: The above program has been executed successfully

Q. Demonstrate RSA in python

Aim: To write code to demonstrate RSA in python.

Program:

p=5

q=11

n=p*q

pn=(p-1)*(q-1)

e=3

d=27

m=9

c=pow(m,e,n)

print("Cipher Text",c)

decryption=pow(c,d,n)

print("decrypted",decryption)

Output:

Conclusion: The above program has been executed successfully

Q. Demonstrate MD5 algorithm in python

Aim: To write code to demonstrate MD5 algorithm in python.

Program:

import hashlib

str1 ="syit class"

md5hashvalue=hashlib.md5()

print(md5hashvalue)

md5hashvalue.update(str1.encode('utf-8'))

messagedigest=md5hashvalue.hexdigest()

messagedigest

print(messagedigest)

Output:

Conclusion: The above program has been executed successfully

Q. Demonstrate RSHash algorithm in python

Aim: To write code to demonstrate RSHash algorithm in python.

Program:

def rshashvalue(str2):

a=63689

b=378551

hashvalue=0

for ch in str2:

hashvalue=hashvalue*a+ord(ch)

a=a*b

return hashvalue & 0xFFFFFFFF

h1=rshashvalue('syit class')

print(h1)

Output:

Conclusion: The above program has been executed successfully

Q. To create, Export and Validate a digital certificate

Aim: To write a code to create, Export & validate a digital certificate

Steps:

- Download and Install OpenSSL

- Add its bin path in “environment variable”

- Open command prompt

Program:

c:\Users\Shreya\Desktop>openssl req -x509 -days 365 -newkey rsa:2048

-keyout private-key.pem -out certificate.pem

Enter PEM Pass: 123456

Country Name (2 letter code) : IN

State or Province Name (full name) (some-state): Maharashtra

Locality Name (L): Mumbai

Organization Name : SIWS

Organization Unit Name: IT

Common Name : TMD

Email Address (E): shreyatiwari@gmail.com

c:\Users\Shreya\Desktop>openssl pkcs12 -export -in certificate.pem

-inkey private-key.pem -out TMD.pfx

Enter pass phrase for private-key.pem: 123456

Enter Export Password: 123456

Verifying - Enter Export Password: 123456

> openssl pkcs12 -in TMD.pfx -clcerts -nokeys -out public-key.pem

Enter Import Password: 123456

>

Import the certificate.

Output:

Conclusion: The above program has been executed successfully